Product Features

- Digital display, one-click pairing, automatic connection, no setup required—simple and quick to use without a phone or computer.

- Supports peer-to-peer and point-to-multipoint bridging, offering flexible applications to meet diverse use cases.

- 5.8 GHz frequency band, supporting spread spectrum technology with a maximum wireless transmission rate of 867 Mbps;

- Exhibits strong anti-interference capability, remains unaffected by external environmental factors, and ensures stable transmission;

- Equipped with MediaTek's engineering-grade chips, delivering stable and reliable performance.

- Featuring a high-power circuit design with an integrated high-gain directional antenna, it delivers strong signal strength and extended transmission range.

- Two 10/100/1000 Mbps adaptive Ethernet ports (both support 24V PoE power supply);

- Supports automatic restart, reduces system operation cache, optimizes system performance, and ensures smooth long-term device operation.

- Supports hiding the bridging SSID from wireless signals to prevent wireless devices from detecting it, protect network data from malicious attacks, and ensure user data security.

- Supports both 24V PoE and 12V DC power supplies with plug-and-play functionality for flexible power sourcing.

- Featuring an outdoor rain-resistant design with IP65 protection rating, it is resistant to rain, dust, wind, and sunlight, effortlessly adapting to various harsh outdoor conditions while ensuring long-term stable operation.

- With a square design that is simple yet elegant, it's easy to install—available for wall mounting or bracket fixation.

- Suitable for various outdoor application scenarios, including construction site monitoring, tower crane monitoring, scenic area monitoring, elevator monitoring, factory premises monitoring, port monitoring, residential community monitoring, and outdoor network transmission.

size of product

| product name | 900M high-power dual-port gigabit digital wireless bridge |  |
| --- | --- | --- |
| product model | WB5891F |  |
| Hardware Specifications |  |  |
| hardware system | Main Chip | MT7621AT+MT7612EN |
|  | internal storage | 128MB |
|  | FLASH | 8MB |
|  | PA | high-power PA |
| Wireless Parameters | protocol standard | Supports IEEE 802.11ac and is backward compatible with IEEE 802.11a/an |
|  | Working Frequency Band | 5.8GHz：5180MHz ~5825MHz |
|  | Wireless Speed | 867Mbps |
|  | transmitting power | 23dBm |
|  | receiving sensitivity | -92dBm |
| physical interface | network interface | Two 10/100/1000 Mbps Ethernet ports (both support 24V PoE power supply) |
|  | Hardware Button | 1 digital button/RESET button |
|  | Power Supply Interface | One DC power port, supporting 12V DC power supply |
|  | Pull the switch | 1 pull-down switch (free switching between terminals A and B) |
|  | Digital Display | 1 digital display (supporting 16 channels from 0 to 9, A to F) |
| Wireless Type | Wireless Type | Built-in 14 dBi, 5.8 GHz bipolar directional antenna |
| pilot lamp | LED pilot lamp | PWR，LAN1，LAN2，4*LINK，WLAN |
| Power Supply/Power Consumption | power supply mode | Supports DC 12V/1A power supply, 24V PoE power supply, and 48V PoE power supply |
|  | Total Power of the Device | ﹤8W |
| Product Accessories | main engine | 900M High-power Dual-Port Gigabit Digital Wireless Bridge Host *1 |
|  | Power Adapter | 24V/1A Gigabit PoE Power Supply |
|  | else | Stainless steel clamp *1 User Manual *1 Qualification Certificate *1 |
| Appearance/Packaging/Weight | Appearance Color | white |
|  | product size | 250mm*90mm*54m |
|  | packing measurement | 272 mm × 235 mm × 73 mm (set of two) |
|  | weight | 2.4 kg (per pack) |
|  | carton size | 570*386*492mm |
|  | packing quantity | 40 pieces per box |
|  | net weight | 48kg |
|  | rough weight | 49.5kg |
| else | The optimal power supply distance for a PoE network cable | Within 65 meters (using Category 5e or higher network cables) |
|  | Bridge Distance | 0–5 km |
|  | IPC Load Capacity | Route 20: No lag, no delay |
|  | way to install | Rod mounting, Wall-mounted installation, Support bracket installation |
|  | levels of protection | IP65 rating; 2 kV lightning protection |
| Software Specifications |  |  |
| Bridge Networking Method | Digital Pairing | Supports digital display and one-click pairing – convenient, simple, and fast |
|  | Manual Pairing | Support (available when digital display function is turned off) |
|  | Point-to-point bridging | support |
|  | Point-to-multipoint bridging | support |
| network | work pattern | AP Mode, WDS Bridge Mode, WDS Repeater Mode, AP-Client Mode, AP-Client+AP Mode |
|  | LAN Configuration | support |
|  | WAN/LAN Flow Rate Control | support |
|  | DHCP server | support |
|  | Network Traffic Monitoring | support |
| Wireless Settings | Wireless Switch | support |
|  | Enable wireless timing | support |
|  | SSID set up | support |
|  | SSID hide | support |
|  | Wireless Security | Supports multiple encryption methods |
|  | Wireless Mode Settings | support |
|  | Bandwidth Settings | support |
|  | Wireless Channel Settings | Supports spread spectrum |
|  | Wireless User Isolation | support |
|  | Wireless Power Regulation | support |
|  | Visitors Network Settings | support |
|  | access control | support |
| system management | Web interface management | support |
|  | user management | support |
|  | firmware upgrade | support |

interface specification

product size

Product Application

I. Point-to-point wireless monitoring transmission applications 1

II. Point-to-point wireless monitoring transmission applications 2

III. Wireless Monitoring Transmission: Point-to-Multipoint Applications 3

IV. Internet of Things Data Transmission Applications

V. Applications of Wireless Data Transmission over the Internet 1

VI. Internet Wireless Transmission Applications 2

VII. Internet Coverage Applications 3