Product Features

- Digital display, one-click pairing, automatic connection, no setup required—simple and quick to use without a phone or computer.

- Supports peer-to-peer and point-to-multipoint bridging, offering flexible applications to meet diverse use cases.

- 5.8 GHz frequency band, supports spread spectrum technology with a maximum wireless transmission rate of 300 Mbps;

- Exhibits strong anti-interference capability, remains unaffected by external environmental factors, and ensures stable transmission;

- Equipped with Qualcomm's engineering-grade chips from the United States, delivering stable and reliable performance.

- Featuring a high-power circuit design with an integrated high-gain directional antenna, it delivers strong signal strength and extended transmission range.

- Supports automatic restart, reduces system operation cache, optimizes system performance, and ensures smooth long-term device operation.

- Supports hiding the bridging SSID from wireless signals to prevent wireless devices from detecting it, protect network data from malicious attacks, and ensure user data security.

- Supports both 24V PoE and 12V DC power supplies with plug-and-play functionality for flexible power sourcing.

- Featuring an outdoor rain-resistant design with IP65 protection rating, it is resistant to rain, dust, wind, and sunlight, effortlessly adapting to various harsh outdoor conditions while ensuring long-term stable operation.

- With an olive-shaped design that is simple yet elegant, it's easy to install and can be mounted either wall-mounted or on a bracket.

- Suitable for various outdoor application scenarios, including construction site monitoring, tower crane monitoring, scenic area monitoring, elevator monitoring, factory premises monitoring, port monitoring, residential community monitoring, and outdoor network transmission.

size of product

| product name | 5.8G High-power Long-range Digital Wireless Bridge |  |
| --- | --- | --- |
| product model | WB5830Y |  |
| Hardware Specifications |  |  |
| hardware system | Main Chip | AR9344 |
|  | internal storage | 64MB |
|  | FLASH | 4MB |
| Wireless Parameters | protocol standard | IEEE 802.11a/n |
|  | Working Frequency Band | 5.8G：5180MHz ~5825MHz |
|  | Wireless Speed | 300Mbps |
|  | transmitting power | 22dBm |
| physical interface | network interface | Two 10/100 Mbps Ethernet ports (both support 24V PoE power supply) |
|  | Hardware Button | 1 digital button/RESET button |
|  | Power Supply Interface | One DC power port, supporting 12V DC power supply |
|  | Pull the switch | 1 pull-down switch (free switching between terminals A and B) |
|  | Digital Display | 1 digital display (supporting 16 channels from 0 to 9, A to F) |
| Wireless Type | Wireless Type | Built-in 14 dBi bipolar directional antenna |
| pilot lamp | LED pilot lamp | In order: POWER, WAN/LAN, WAN/LAN, four signal lights, WIFI |
| Power Supply/Power Consumption | power supply mode | Supports DC 12V/1A power supply, 24V PoE power supply, and 48V PoE power supply |
|  | Total Power of the Device | ﹤6W |
| Product Accessories | main engine | 5.8G High-power Long-range Digital Wireless Bridge Host *1 |
|  | Power Adapter | 24V 0.5A PoE power supply |
|  | else | Stainless steel clamp *1 User Manual *1 Qualification Certificate *1 |
| Appearance/Packaging/Weight | Appearance Color | white |
|  | product size | Φ200mm*131mm（Φ*H） |
|  | packing measurement | 313*238*73 mm |
|  | Weight per unit | 0.89KG |
|  | carton size | 495*385*333 mm |
|  | packing quantity | 10 pieces per box |
|  | net weight | 8.9KG |
|  | rough weight | 10.2KG |
| else | The optimal power supply distance for a PoE network cable | Within 65 meters (using Category 5e or higher network cables) |
|  | Bridge Distance | 0–3 km |
|  | IPC Load Capacity | Route 15: No lag, no delay |
|  | way to install | Rod mounting, Wall-mounted installation, Support bracket installation |
|  | levels of protection | IP65 rating; 2 kV lightning protection |
| Software Specifications |  |  |
| System Operation Mode | Normal Mode | This mode supports AP mode, client mode, and relay mode. Support manual pairing bridging (in this mode, the digital display function is disabled) |
|  | Digital Display Mode | Auto-pairing supports one-click digital pairing without entering the web interface for manual pairing |
| Bridge Networking Method | Digital Pairing | Supports digital display and one-click pairing – convenient, simple, and fast |
|  | Manual Pairing | support |
|  | Point-to-point bridging | support |
|  | Point-to-multipoint bridging | support |
| network | work pattern | Wireless Bridge, AP Mode, Client Mode, Relay Mode |
|  | LAN Configuration | support |
|  | DHCP server | support |
|  | Wireless Network Status Display | support |
| Wireless Settings | SSID revise | support |
|  | SSID hide | support |
|  | way of encryption | WPA-PSK，WPA2-PSK |
|  | Bandwidth Settings | support |
|  | Wireless Channel Modification | Supports spread spectrum |
|  | Wireless User Isolation | support |
|  | Wireless Power Regulation | support |
|  | multi SSID | support |
| system management | Web interface management | support |
|  | SL | Supports Chinese/English |
|  | softdog | support |
|  | Reset定时 restart | support |
|  | user management | support |
|  | firmware upgrade | support |
|  | Firmware File Import and Export | support |

interface specification

product size

Product Application

I. Point-to-point wireless monitoring transmission applications 1

II. Point-to-point wireless monitoring transmission applications 2

III. Wireless Monitoring Transmission: Point-to-Multipoint Applications 3

IV. Internet of Things Data Transmission Applications

V. Applications of Wireless Data Transmission over the Internet 1

VI. Internet Wireless Transmission Applications 2

VII. Internet Coverage Applications 3