specification

Model： WB860H-32

- Brand and Manufacturer Information

Brand: Neutral

LOGO： neutral

- Model and Version

Model: MWS-WB860H-32

Version: V1.0

- Product Overview and Features

Product Overview

The MWS-WB860H-32 is a high-performance, high-speed industrial-grade long-range wireless bridge featuring extensive transmission range, strong signal penetration capability, and excellent interference resistance.

product features

- The use of specialized TVS lightning protection devices enhances the product's performance under harsh environmental conditions.

- Equipped with an external 32 dBi bipolar directional antenna, with a maximum coverage range of up to 35 kilometers.

- Supports both 24–48 V PoE power supply and 12 V DC power supply.

- TDMA Intelligent Dynamic Polling Protocol

- Supports ATPC power control

- Supports IPv6

- technical parameter

| Specification Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| product model | WB860H-32 |  |  |  |  |
| CPU | QCA9563+QCA9882+QCA8334 |  |  |  |  |
| Flash | SPI NOR 16MB |  |  |  |  |
| DDR | DDR2-128MB |  |  |  |  |
| 5G Operating Frequency Bands | 5.150GHz～5.850GHz |  |  |  |  |
| 5G Wi-Fi Transmission Protocol | 802.11 a / n /ac |  |  |  |  |
| top speed | 867Mbps |  |  |  |  |
| antenna | Two N-F connectors, external parabolic antenna, gain: 32 dBi |  |  |  |  |
| 5 GHz wireless power | 802.11a | 54M | 23±2dBm | 6M | 26±2dBm |
|  | 802.11n HT20 | MCS7 | 22±2dBm | MCS0 | 26±2dBm |
|  | 802.11n HT40 | MCS7 | 22±2dBm | MCS0 | 26±2dBm |
|  | 802.11ac VHT20 | MCS9 | 21±2dBm | MCS0 | 26±2dBm |
|  | 802.11ac VHT40 | MCS9 | 21±2dBm | MCS0 | 26±2dBm |
|  | 802.11ac VHT80 | MCS9 | 21±2dBm | MCS0 | 26±2dBm |
| 5 GHz reception sensitivity | 802.11a | 54M | -75dBm | 6M | -92dBm |
|  | 802.11n HT20 | MCS7 | -72dBm | MCS0 | -90dBm |
|  | 802.11n HT40 | MCS7 | -69dBm | MCS0 | -88dBm |
|  | 802.11ac VHT20 | MCS9 | -65dBm | MCS0 | -88dBm |
|  | 802.11ac VHT40 | MCS9 | -62dBm | MCS0 | -86dBm |
|  | 802.11ac VHT80 | MCS9 | -59dBm | MCS0 | -84dBm |
| 5G EVM | 802.11a_54M: ≤-25 dB；802.11n_MCS7: ≤-28 dB；802.11AC_MCS9:≤-32dB |  |  |  |  |
| frequency deviation （ppm） | ±20ppm |  |  |  |  |
| Interface (WAN) | One 10/100/1000 Mbit/s adaptive WAN port, supporting 24–48 V POE power supply |  |  |  |  |
| Interface (LAN) | 1 adaptive LAN port with speeds of 10 Mbps, 100 Mbps, or 1000 Mbps |  |  |  |  |
| supply electricity | Supports PoE 24V Passive and DC 12–24V; customizable with a 48V PoE version compliant with IEEE 802.3af standards |  |  |  |  |
| maximum power dissipation | < 15W |  |  |  |  |
| RST key | Reset Key (hold for 6–10 seconds to restore factory settings) |  |  |  |  |
| State Indicator Light | SYS light, WAN port light, LAN port light, signal lights ×5 |  |  |  |  |
| product size | 243mm*218mm*67mm |  |  |  |  |
| weight | 1.756 kg (main unit), 2.984 kg (gross weight) |  |  |  |  |
| work environment | Normal operating temperature: -40°C to 70°C; Maximum operating temperature: -40°C to 70°C Storage temperature: -40°C to 70°C; Humidity: 5% to 95% (no condensation) |  |  |  |  |
| levels of protection | IP67 |  |  |  |  |
| ESD | Air discharge: ±15 K; Contact discharge: ±8 K |  |  |  |  |
| surge | Common-mode 4K, differential-mode 2K |  |  |  |  |

- software function

| Wireless Mode | Access Point (Automatic WDS), Access Point (TDMA3), Site (WDS/TDMA3), Site (ARPNAT) |
| --- | --- |
| Wireless Technology | Smart dynamic polling, automatic channel selection, automatic modulation method selection, and Automatic Transmission Power Control (ATPC) |
| Wireless Security | WPA/WPA2 Personal Encryption, WPA/WPA2 Enterprise Encryption, WACL, User Isolation |
| wireless QoS | TDMA implements 4-channel priority |
| Network Mode | IPv4 to IPv6 Bridging |
| network technique | Supports VLAN and VLAN forwarding |
| WAN protocol | Static IP, DHCP Client |
| serve | DHCP service, SNMP service, NTP client, route tracking, ping watchdog |
| manage | HTTP(S) GUI, SSH, SNMP read, WNMS, Telnet |
| tool | Site Survey, Connection Test, Antenna Calibration |

- Interface Diagram

- Product Size Diagram

- Antenna Specifications

| External Antenna |  |
| --- | --- |
| Working Frequency Band | 5180 ~5850 MHz |
| gain | 32 dBi |
| Polarization Method | ±45° bipolarization |
| Cross-polarization isolation degree | 30 dBi |
| Maximum Standing Wave Ratio | <1.8 |
| Horizontal flap angle | 3.4 ° |
| Vertical flap angle | 3.4 ° |
| rough weight | 14kg/2pcs |

- Attachment List

Host ×1; Ethernet cable ×1; PoE module ×1; Power cord ×1; Ethernet port interfaces ×2; L-shaped bracket ×1; Installation kit ×3; User manual ×1; Certificate of conformity and warranty card ×1