Product Features

- Digital display, one-click pairing, automatic connection, no setup required—simple and quick to use without a phone or computer.

- Supports peer-to-peer and point-to-multipoint bridging, offering flexible applications to meet diverse use cases.

- 5.8 GHz frequency band, supports spread spectrum technology with a maximum wireless transmission rate of 12 0 1  Mbps;

- High anti-interference capability, unaffected by external environmental factors, and stable transmission;

- Equipped with MediaTeks engineering-grade chips, delivering stable and reliable performance.

- Featuring a high-power circuit design with an integrated high-gain directional antenna, it delivers strong signal strength and extended transmission range.

- Two 10/100/1000 Mbps adaptive Ethernet ports (both support 24V PoE power supply);

- Supports automatic restart, reduces system operation cache, optimizes system performance, and ensures smooth long-term device operation without lag.

- Supports hiding bridging SSID wireless signals to prevent wireless devices from detecting them, protect network data from malicious attacks, and ensure user data security.

- Supports 24V PoE, 48V PoE, and 12V DC power supplies with plug-and-play functionality for flexible power sourcing.

- Featuring an outdoor rain-resistant design with IP65 protection rating, it is resistant to rain, dust, wind, and sunlight, effortlessly adapting to various harsh outdoor conditions while ensuring long-term stable operation.

- With a square design that is simple yet elegant, its easy to install—available for wall mounting or bracket fixation.

- Suitable for various outdoor application scenarios, including construction site monitoring, tower crane monitoring, scenic area monitoring, elevator monitoring, factory premises monitoring, port monitoring, residential community monitoring, and outdoor network transmission.

size of product

| product name | WIFI6 High-Power Digital Wireless Bridge |  |
| --- | --- | --- |
| product model | WB5896G |  |
| Hardware Specifications |  |  |
| hardware system | Main Chip | MT 7621A T+ MT7975+ MT7905 |
|  | internal storage | DDR3 256 MB |
|  | FLASH | 16 MB |
|  | PA | high-power PA |
| Wireless Parameters | protocol standard | Supports IEEE 802.11a x/ac/an/a |
|  | Working Frequency Band | 5.8GHz：5180MHz ~5825MHz |
|  | Wireless Speed | 1201 Mbps |
|  | transmitting power | 2 3 dBm |
|  | receiving sensitivity | -9 2 dBm |
| physical interface | network interface | Two Ethernet ports (10/100/1000 Mbps, both supporting 24V PoE power supply) |
|  | Hardware Button | 1 digital button/RESET button |
|  | Power Supply Interface | One DC power port, supporting 12V DC power supply |
|  | Pull the switch | 1 pull-down switch (A and B terminals can be freely switched) |
|  | Digital Display | 1 digital display (supporting 16 channels from 0 to 9, A to F) |
| Wireless Type | Wireless Type | Built-in 1×4-dBi 5.8 GHz dipolar directional antenna |
| pilot lamp | LED pilot lamp | P WR， LAN1， LAN2， WIFI， LINK |
| Power Supply/Power Consumption | power supply mode | Supports DC12V 1A power supply, 24V PoE power supply, and 48V PoE power supply |
|  | Total Power of the Device | ﹤ 8 W |
| Product Accessories | main engine | WIFI6 High-Power Digital Wireless Bridge *1 |
|  | Power Adapter | 24V/1A Gigabit PoE power supply |
|  | else | User Manual *1, Certificate of Compliance *1, Stainless Steel Clasp *1 |
| Appearance/Packaging/Weight | Appearance Color | white |
|  | product size | 223mm*223mm*110mm |
|  | packing measurement |  |
|  | Weight per unit |  |
|  | carton size |  |
|  | packing quantity |  |
|  | net weight |  |
|  | rough weight |  |
| else | The optimal power supply distance for a PoE network cable | Within 65 meters (using Category 5e or higher network cables) |
|  | Bridge Distance | 0–5 km |
|  | IPC Load Capacity | Route 20: No lag, no delay |
|  | way to install | Rod mounting, Wall-mounted installation, Support bracket installation |
|  | levels of protection | IP65 rating; lightning protection rated at 2 KV |
| Software Specifications |  |  |
| Bridge Networking Method | Digital Pairing | Supports digital display and one-click pairing – convenient, simple, and fast |
|  | Manual Pairing | Support (available when digital display function is turned off) |
|  | Point-to-point bridging | support |
|  | Point-to-multipoint bridging | support |
| network | work pattern | AP Mode, WDS Bridge Mode, WDS Repeater Mode, AP-Client Mode, AP-Client+AP Mode |
|  | LAN Configuration | support |
|  | WAN/LAN Flow Rate Control | support |
|  | DHCP server | support |
|  | Network Traffic Monitoring | support |
| Wireless Settings | Wireless Switch | support |
|  | Enable wireless timing | support |
|  | SSID set up | support |
|  | SSID hide | support |
|  | Wireless Security | Supports multiple encryption methods |
|  | Wireless Mode Settings | support |
|  | Bandwidth Settings | support |
|  | Wireless Channel Settings | Supports spread spectrum |
|  | Wireless User Isolation | support |
|  | Wireless Power Regulation | support |
|  | Visitors Network Settings | support |
|  | access control | support |
| system management | Web interface management | support |
|  | user management | support |
|  | firmware upgrade | support |

interface specification

product size

Product Application

I. Point-to-point wireless monitoring transmission applications 1

II. Wireless Monitoring Transmission Point-to-Point Applications 2

III. Wireless Monitoring Transmission: Point-to-Multipoint Applications 3

IV. Internet of Things Data Transmission Applications

V. Internet Wireless Transmission Applications 1

VI. Internet Wireless Transmission Applications 2

VII. Internet Coverage Applications 3