Product Characteristics

WA580 is a high-performance, high-rate 11ax indoor wall-mounted wireless AP that provides 2.4G and 5.8GHz dual-band wireless high-rate service, with 2.4G providing up to 574Mbps access rate and 5G providing up to 1201Mbps access rate.

Product Features

- Adopts standard type 86 bottom box into the wall installation, flexible and universal, simple and convenient installation;

- 2.4GHz band compatibility, long transmission distance, 5GHz band interference is small, dual frequency concurrent, wireless transmission rate up to 1800Mbps, better meet the crowded places of wireless coverage needs;

- Support seamless roaming, more smooth switching, improve each user's wireless experience;

- Support automatic channel optimization, reduce mutual interference between channels, optimize the overall performance of wireless networks;

- Built-in 2 2.4G antennas, 2 5G antennas, improve the wireless signal quality of AP coverage area;

- Fat and thin, support AC controller unified management, one-click delivery configuration, saving labor maintenance costs;

- Support 802.3af protocol, standard 48V PoE power supply, plug and play, more flexible and convenient wiring;

- The machine capacity can reach 100+;

- Suitable for all kinds of indoor scenes, such as: hotel, guesthouse, coffee shop, office, school, shopping mall and other application scenarios.

Product Specifications

| Product Name | AX1800 WIFI6 Dual-band In-wall Wireless AP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Product Model | WA580 |  |  |  |  |  |
| Hardware Specifications |  |  |  |  |  |  |
| CPU | MT7621DAT+MT7905+MT7975 |  |  |  |  |  |
| DDR3 | 128MB built in |  |  |  |  |  |
| FLASH | SPI NOR 16MB |  |  |  |  |  |
| Wireless Protocol Standards | 2.4G: Supports IEEE 802.11b /g/n/ax 5G: Supports IEEE 802.11a /n/ac/ax |  |  |  |  |  |
| Wireless Operating Bands | 2.4GHz: 2.412GHz - 2.472GHz 5GHz: 5.180GHz - 5.825GHz |  |  |  |  |  |
| Wireless Maximum Speed | 2.4G offers a maximum access rate of 574Mbps 5.8G offers a maximum access rate of 1201Mbps |  |  |  |  |  |
| 2.4G Transmit Power | 802.11 b | 11M |  | 17±2dBm | 1M | 18±2dBm |
|  | 802.11 g | 54M |  | 16±2dBm | 6M | 17±2dBm |
|  | 802.11n HT20 | MCS7 |  | 16±2dBm | MCS0 | 17±2dBm |
|  | 802.11n HT40 | MCS7 |  | 15±2dBm | MCS0 | 16±2dBm |
|  | 802.11ax HE20 | MCS11 |  | 15±2dBm | MCS0 | 16±2dBm |
|  | 802.11ax HE40 | MCS11 |  | 15±2dBm | MCS0 | 16±2dBm |
| 5G Transmit Power | 802.11 a | 54M |  | 16±2dBm | 6M | 17±2dBm |
|  | 802.11n HT20 | MCS7 |  | 15±2dBm | MCS0 | 16±2dBm |
|  | 802.11n HT40 | MCS7 |  | 15±2dBm | MCS0 | 16±2dBm |
|  | 802.11ac HT20 | MCS7 |  | 15±2dBm | MCS0 | 16±2dBm |
|  | 802.11ac HT40 | MCS7 |  | 15±2dBm | MCS0 | 16±2dBm |
|  | 802.11ac HT80 | MCS9 |  | 14±2dBm | MCS0 | 15±2dBm |
|  | 802.11aX HT20 | MCS11 |  | 15±2dBm | MCS0 | 16±2dBm |
|  | 802.11aX HT40 | MCS11 |  | 15±2dBm | MCS0 | 16±2dBm |
|  | 802.11aX HT80 | MCS11 |  | 14±2dBm | MCS0 | 15±2dBm |
| 2.4G Receiving Sensitivity | 802.11 b | 11M |  | -85dBm | 1M | -90dBm |
|  | 802.11 g | 54M |  | -72dBm | 6M | -90dBm |
|  | 802.11n HT20 | MCS7 |  | -70dBm | MCS0 | -87dBm |
|  | 802.11n HT40 | MCS7 |  | -68dBm | MCS0 | -85dBm |
|  | 802.11ax HT20 | MCS11 |  | -60dBm | MCS0 | -83dBm |
|  | 802.11ax HT40 | MCS11 |  | -56dBm | MCS0 | -83dBm |
| 5G Receiving Sensitivity | 802.11 a | 54M |  | -72dBm | 6M | -91dBm |
|  | 802.11n HT20 | MCS7 |  | -70dBm | MCS0 | -90dBm |
|  | 802.11n HT40 | MCS7 |  | -68dBm | MCS0 | -88dBm |
|  | 802.11ac HT20 | MCS7 |  | -70dBm | MCS0 | -90dBm |
|  | 802.11ac HT40 | MCS7 |  | -68dBm | MCS0 | -88dBm |
|  | 802.11ac HT80 | MCS9 |  | -58dBm | MCS0 | -85dBm |
|  | 802.11ax HT20 | MCS11 |  | -62dBm | MCS0 | -88dBm |
|  | 802.11ax HT40 | MCS11 |  | -58dBm | MCS0 | -86dBm |
|  | 802.11ax HT80 | MCS11 |  | -55dBm | MCS0 | -84dBm |
| 2.4G EVM | 802.11b; ≤-10dB; 802.11g; ≤-25dB; 802.11n; ≤-28dB; 802.11ax; ≤-35dB |  |  |  |  |  |
| 5G EVM | 802.11a; ≤-25dB; 802.11n; ≤-28dB; 802.11ac; ≤-32dB; 802.11ax; ≤-35dB |  |  |  |  |  |
| Frequency Offset (ppm) | ±20 ppm |  |  |  |  |  |
| 2.4G Antenna | 2 built-in antennas, gain: 3dBi |  |  |  |  |  |
| 5G Antenna | 2 built-in antennas, gain: 3dBi |  |  |  |  |  |
| Maximum number of wireless accesses supported | 100 + |  |  |  |  |  |
| Network Interface | Back of the device: 1 10/100/1000M adaptive RJ45 port with 48V PoE power supply Front of the device: 1 10/100/1000M adaptive RJ45 port |  |  |  |  |  |
| Reset Switch | 1 RESET button (hold down for 10-15 seconds to restore the default factory settings) |  |  |  |  |  |
| Status Indicator | SYS indicator light |  |  |  |  |  |
| Power Supply Mode | Standard PoE power supply Support 802.3af protocol |  |  |  |  |  |
| Maximum Power Consumption | < 14 w |  |  |  |  |  |
| Product Size | 86mm×86mm×40mm |  |  |  |  |  |
| Weight | < 0.35 kg |  |  |  |  |  |
| Surge | Common mode 1K, difference mode 0.5K |  |  |  |  |  |
| Exterior Material | Imported high quality high temperature resistant PC material + aluminum high efficiency heat dissipation material |  |  |  |  |  |
| Appearance Color | White |  |  |  |  |  |
| Protection Level | IP31 |  |  |  |  |  |
| Installation Method | In-wall installation |  |  |  |  |  |
| Product Accessories | AX1800 WIFI6 Dual-band In-wall Wireless AP *1 Quick Installation Wizard *1, Warranty card *1, Certificate of conformity *1 |  |  |  |  |  |
| Use Environment | Working temperature: -10℃ ~ 55℃ Storage temperature: -40℃ ~ 70℃ Working humidity: 10% ~ 90%RH non-condensing Storage humidity: 5% ~ 95%RH non-condensing |  |  |  |  |  |
| Software Specifications |  |  |  |  |  |  |
| Working Mode | Fat and thin in one, Thin AP mode (FIT AP) : Unified management by the wireless controller (AC) FAT AP mode (FAT AP) : Independent web page management |  |  |  |  |  |
| AC Unified Management | Support |  |  |  |  |  |
| Wireless Capabilities | SSID broadcast |  | Support |  |  |  |
|  | SSID Hiding |  | Support |  |  |  |
|  | Chinese SSID |  | Support |  |  |  |
|  | Multiple SSIDs |  | Support |  |  |  |
|  | SSID Merge |  | Support 2.4GHz, 5.8GHz multiple bands combined into a common SSID |  |  |  |
|  | Wireless encryption |  | WPA, WPA2, WPA-PSK, WPA2-PSK |  |  |  |
|  | User isolation |  | Isolation between wireless networks and internal aps |  |  |  |
|  | Whitelist management |  | Supports MAC address whitelist management |  |  |  |
|  | Transmit power setting |  | Support 1dBm linear adjustment |  |  |  |
|  | VLAN settings |  | Support SSID and VLAN binding |  |  |  |
|  | Limit the number of wireless clients |  | Support |  |  |  |
|  | Seamless roaming |  | Support |  |  |  |
| Maximum number of access users | 2.4 G |  | 50 |  |  |  |
|  | 5G |  | 50 |  |  |  |
| Systems Administration | Equipment management |  | All Chinese WEB management |  |  |  |
|  | System log |  | Support |  |  |  |
|  | Config down |  | Support |  |  |  |
|  | Remote reboot |  | AC remote restart is supported |  |  |  |
|  | Factory data reset. |  | Support |  |  |  |
|  | Backup configuration |  | Support |  |  |  |
|  | Import configuration |  | Support |  |  |  |
|  | Software upgrades |  | Support |  |  |  |
|  | Ping Watchdog |  | Support |  |  |  |

Product networking and application

- Gateway Mode

- Bypass Mode