Product Characteristics

- Six external 3DB omnidirectional antennas;

- Support relay mode, AP mode;

- Built-in 100~240V 50/60Hz AC power module, no external power adapter;

- There are multiple specification standard plugs available for selection, supporting the Chinese standard, European standard, American standard, British standard, etc.

- 1200Mbps transmission rate, dual-band, at the same time support 2.4GHz &5.8GHz frequency band, anti-interference ability is stronger, wireless transmission faster and more stable;

- The device is equipped with two 100M RJ45 ports. It supports the conversion from wireless to wired connection and can be connected to desktop computers, laptops, etc. for wired Internet access.

- Professional intelligent WiFi relay, support WiFi hotspot automatic scanning, and WiFi hotspot memory function;

- Support WEB background Chinese-English dual language interface, one-click setting wizard, quick and simple configuration;

- With good compatibility, compatible with all smart phones, tablets, laptops and other WiFi devices.

Product Specifications

| Product Name |  | 1200M Dual-band Wireless Repeater |  |
| --- | --- | --- | --- |
| Product Model |  | Z1200C |  |
| Hardware Specifications |  |  |  |
| Hardware System | Main chip |  | MT7620A+MT7612EN |
|  | Memory |  | DDR2 64MB |
|  | FLASH |  | 8MB |
| Wireless Parameters | Protocol standards |  | Support IEEE 802.11ac while backward compatible with IEEE 802.11a/b/g/n |
|  | Operating frequency band |  | Dual band, support both 2.4GHz and 5.8GHz (11AC) |
|  | Wireless rate |  | 1200Mbps(2.4 GHz：300 Mbps + 5.8 GHz：867 Mbps) |
|  | Transmit power |  | 2.4 GHz：18dBm ± 1 dBm 5.8 GHz：18dBm ± 1 dBm |
|  | Antenna gain |  | 3dBi omnidirectional antenna *6 |
|  | Reception sensitivity |  | -90dBm ± 2dBm |
| Physical Interface | Network interface |  | 10/100Mbps LAN port *2 |
|  | Button |  | 1 reset key /WPS key |
| Indicator Light | LED indicator |  | PWR/WIFI/Repeater/LAN/LAN in that order |
| Power Supply/ Power Consumption | Mode of power supply |  | AC 100~240V 50/60Hz |
|  | Plug-type |  | Optional, support China, European, British, US and other specifications,etc |
|  | Power of the machine |  | < 5 w |
| Product Accessories | Device host |  | 1200M dual band repeater host *1 |
|  | Other |  | User Manual *1, Certificate *1 |
| Appearance/Packaging | Product dimensions |  | 180mm*84mm*34mm (L*W*H, does not include plug size) |
|  | Appearance color |  | White |
|  | Weight |  | 100g |
|  | Package size |  | 195*90*70mm（L*W*H） |
| Other | Covered area |  | 0 - 80㎡ |
|  | Use environment |  | Working temperature: -10℃ ~ 55℃ Storage temperature: -40℃ ~ 70℃ Working humidity: 10% ~ 90%RH non-condensing Storage humidity: 5%-95%RH Non-condensing |
| Software Specifications |  |  |  |
| Network | Work mode |  | Relay mode, AP mode |
| Wireless Security | Wireless security |  | Supports WPA2, WPA, WPA2-PSK AES, WEP encryption |
| Other | Product features |  | Professional intelligent WIFI repeater, support hotspot automatic scanning, support WIFI hotspot memory function |
|  |  |  | At the same time support 2.4GHz and 5.8GHz two bands of the network, the signal coverage is wider, the transmission rate is faster and more stable, and the anti-interference ability is stronger |
|  | Number of user connections |  | Maximum number of user connections is 35 |
|  | System language |  | Chinese-english dual language adaptive |
|  | Management mode |  | Web Configuration Management |
|  | Applicable scenarios |  | Multi-home, duplex, cross-floor, rental house, apartment, villa, etc |

Product size: