Product Characteristics

- Industrial design, equipped with all Netcom 4G module, support Mobile, Unicom, Telecom operators to choose;

- 4G to wired to WIFI signal, to meet various application scenarios;

- Large number of tape machines, easy access to a variety of wireless terminal equipment;

- No wiring, say goodbye to wired broadband, flexible installation;

- Free to change the card, plug and play, the card has a network;

- External two 2.4G,two 4G high-gain omnidirectional antennas, strong penetration, stable signal.

Product Specifications

| Product Name | Industrial-grade Smart 4G Router |  |
| --- | --- | --- |
| Product Model | RG300B |  |
| Hardware Specifications |  |  |
| Hardware System | Master control | MT7628DAN |
|  | FLASH | 8MB |
|  | 4G module | EC200A-CN 4G Full Netcom (optional) |
| Wireless Parameters | Protocol standards | IEEE 802.11b/g/ n |
|  | Operating frequency band | 2.4GHz: 2412MHz to 2472MHz |
|  | Wireless rate | 300Mbps Max |
|  | 4G speed | LTE-FDD: Max downlink speed 150Mbps, Max uplink speed 50Mbps LTE-TDD: Max downlink speed 130Mbps, Max uplink speed 30Mbps |
|  | Transmit power | 18dBm Max |
| Physical Interface | Network interface | 1 10/100 Mbps RJ45 Ethernet WAN port 4 10/100 Mbps RJ45 Ethernet LAN ports |
|  | Power port | 1 DC power port, support 12V DC power supply |
|  | SIM card port | 1 standard self-ejecting Mini SIM card slot |
|  | Key switch | 1 RESET button |
|  | PCI-E interface | 1 built-in PCI-E interface (optional or replacement of different brands of 4G modules) |
|  | Antenna interface | 2 4G antennas Standard SMA interface (external interface, can be replaced with different antennas) 2 WIFI antenna standard SMA interface (external interface, can be replaced with different antennas) |
| Indicator Light | LED indicator | In order: POWER, 4G, WIFI, WAN, LAN1, LAN2, LAN3, LAN4 |
| Power supply/ Power consumption | Mode of power supply | DC 12V ~ 1A power supply |
|  | Machine power | < 6 w |
| Product Accessories | Mainframe | Industrial-grade Smart 4G Router host *1 |
|  | Power adapter | DC 12V/1A |
|  | Antennas | 2 5dBi high gain omnidirectional WIFI antennas 2 5dBi high gain omnidirectional 4G antennas |
|  | Other | User Manual *1, Warranty card *1, Certificate *1 |
| Appearance/Packaging | Product dimensions | Overall dimensions (L * W * H) : 130mm*83mm*30mm (without antenna) Antenna height: 172mm |
|  | Appearance Material | Iron shell with frosted finish |
|  | Exterior color | Black (appearance color can be customized) |
|  | Weight | < 1.5 kg |
| Other | Level of protection | IP31, 2KV lightning protection |
|  | Installation method | Desktop, wall hanging |
|  | Use environment | Working temperature: -10℃ ~ 55℃ Storage temperature: -40℃ ~ 70℃ Working humidity: 10% ~ 90%RH non-condensing Storage humidity: 5% ~ 95%RH non-condensing |
| Software Specifications |  |  |
| Network | WAN access method | Wired, 4G |
|  | Extranet address detection | Support |
|  | Extranet status display | Support |
|  | LAN setup | Support |
|  | DHCP server | Support |
|  | Uplink/downlink rate display | Support |
| 4G dialing | 4G dialing method | Auto/manual |
|  | MAC address clone | Support |
|  | 4G signal strength display | Support |
|  | 4G module IMEI recognition | Support |
|  | SIM card number display | Support |
| Wireless Settings | Wireless switch | Support |
|  | SSID Setting | Support |
|  | SSID Hiding | Support |
|  | User isolation | Support |
|  | Wireless channel setup | Support |
|  | Transmit power Setting | Support |
|  | Encryption method | WPAPSK, WPA2PSK, WPAPSKWPA2PSK |
|  | Type of encryption | TKIP, AES, TKIPAES |
|  | Visitor Control | Support |
|  | QoS Settings | Support |
| Advanced Settings | Static routing | Support |
|  | Firewall | Support DMZ setup, port mapping, terminal management, MAC filtering, etc |
|  | System administration | Support firmware upgrade, scheduled restart, firmware file import and export, etc |
|  | System tools | PING detection, route tracing, DHCP detection |

Interface Description

1. Front Interface

2. Side Interface

3. Packing List