16+2+1F Full Gigabit Smart POE Switch

DATA SHEET

【Product Overview】

MWS-SG1621Q series unmanaged PoE switches have 18 Gigabit Ethernet ports + 1 Gigabit SFP optical port, of which 1-16 ports support IEEE 802.3af/at standard PoE power supply, and the single-port PoE power reaches 30W. MWS-SG1621Q service ports all support 4KV electrostatic lightning protection, and the downlink ports support POE watchdog automatic restart troubleshooting function, port VLAN isolation function, and one-key 10M/250m long-distance transmission mode (1-8 ports) and other practical function switches, making use, operation and maintenance simpler and more efficient. As a PoE power supply device, it can automatically detect and identify the powered device that meets the standard and supply it with network cables. It can supply power to POE terminal equipment such as wireless APs, webcams, IP phones, and video access control intercoms through network cables, meeting the network environment that requires high-density PoE power supply, and is suitable for hotels, campuses, campuses, communities, and small and medium-sized enterprises to build cost-effective networks.

Non-network management model, plug and play, no configuration, easy to use.

# 【Product Feature】

- Single-port PoE power up to 30W, af/at adaptive, widely compatible with a variety of network terminals;

- Photoelectric hybrid uplink port, flexibly adapted to network cable and optical fiber connection, suitable for various networking expansion and expansion, easier networking;

- 1~8 ports supports 10M/250m long-distance transmission, automatically optimises the network, and easily solves long-distance point communication and power supply problems;

- 1~16 ports support PoE watchdog function, automatic restart, intelligent troubleshooting, reducing the number of visits for maintenance and lowering after-sales O&M costs;

- Supports port VLAN to suppress network storms and improve network transmission stability;

- Enterprise-class high-bandwidth switching chip, high backplane, large cache, reduce network congestion;

- Supports IEEE802.3X full duplex intelligent flow control for lag-free HD transmission of large bit streams;

- Supports MAC address auto learning and updating, and port auto flip (Auto MDI/MDI-X) function;

- The service port supports 4KV anti-surge (lightning) protection, PoE short-circuit and overload protection for safer and more secure use;

- Steel forging metal shell, shielding interference, strong heat dissipation, strong, rust-proof more durable;

- Plug and play, no configuration required, easy and simple to use.

# 【Technical Specifications】

| Model |  | MWS-SG1621Q |  |  |
| --- | --- | --- | --- | --- |
| Port Characteristics |  |  |  |  |
| Fixed Port | Downlink port | 16* 10/100Base-TX RJ45 network port |  |  |
|  | Uplink port | 2* 10/100/1000Base-T RJ45 network port 1* 1000M SFP optical port slot |  |  |
|  | PoE port | 1-16 support af/at standard PoE power supply, transmission distance 100 meters |  |  |
| Port Characteristics |  | 1-18 port 10/100/1000M adaptive, full/half duplex MDI/MDI-X adaptive |  |  |
| Twisted Pair Transmission |  | 10BASE-T：Cat3,4,5 UTP(≤100 meters) 100BASE-TX：Cat5 or later UTP(≤100 meters) 1000BASE-T：Cat5e or later UTP(≤100 meters) |  |  |
| Optical Port Characteristics |  | Gigabit SFP fibre optic interface, default does not support optical module, need to purchase separately, (optional order mode/multimode, single/dual fibre optical module, LC/SC) |  |  |
| Fibre-optic Cable |  | Multimode ：850nm 0～550M ， 50μm/125μm&62.5μm/125μm Single mode ：1310nm 0～40KM ， 1550nm 0～120km, 9.5μm/125μm |  |  |
| Function Switch |  | Tranche1：VLAN |  | Toggle to "ON" to realise the function of power supply port isolation, default "OFF" |
|  |  | Tranche2：PoE Watchdog |  | Toggle to"ON" to enable the PoE auto restart function of the power supply port, default "OFF" |
|  |  | Tranche3：Extend |  | Toggle to "ON" to enable (1-8 power supply ports) 10M/ 250m long distance function, default "OFF" |
| Switching Chip Parameter |  |  |  |  |
| Network Management Type |  | Unmanaged switch |  |  |
| Networking Protocol |  | IEEE802.3 10BASE-T； IEEE802.3i 10Base-T； IEEE802.3u 100Base-TX； IEEE802.3x Flow Control； IEEE802.3ab 1000Base-T； IEEE802.3z 1000Base-X； IEEE 802.3af DTE Power via MDI； IEEE 802.3at DTE Power via MDI； |  |  |
| Transfer Mode |  | Store-and-forward（top speed） |  |  |
| Backplane Bandwidth |  | 52Gbps（non-obstructive） |  |  |
| Packet Forwarding Ratio@64byte |  | 28.27Mpps |  |  |
| MAC Address Table |  | 8K |  |  |
| Packet Forwarding Cache |  | 4Mbits |  |  |
| Giant Frame |  | 10Kbytes |  |  |
| Indicator Light |  | Power indicator: PWR (green); Network indicator: 1-18 (green); Optical port indicator : 19(green). |  |  |
| MTBF |  | 100,000 hours |  |  |
| POE and Power Characteristics |  |  |  |  |
| POE Port |  | 1-16 ports support IEEE802.3af/at standard POE power supply |  |  |
| Power Supply Pins |  | Default 1/2(+), 3/6(-) 4-pole power supply |  |  |
| Maximum PoE power per port |  | 30W, standard IEEE802.3af/at |  |  |
| Total Power/ Input Voltage |  | 300W （AC100-240V） |  |  |
| Power Consumption |  | Standby power consumption: <15W; Full load power consumption: <250W |  |  |
| Matching power supply |  | Built-in switching power supply AC：100～240V 50-60Hz 5.0A |  |  |
| Physical Specification Parameters |  |  |  |  |
| Operating Temperature/Humidity |  |  | -20～+55°C；5%～90% RH,non-condensing |  |
| Storage Temperature/Humidity |  |  | -40～+75°C；5%～95% RH,non-condensing |  |
| Appearance Size (L*W*H) |  |  | 310mm*205mm*44mm |  |
| Net/Gross weight |  |  | 1.6kg / 2.14kg |  |
| Installation Method |  |  | Desktop, rack mount |  |
| Product Certification And Warranty |  |  |  |  |
| Lightning Protection / Protection Level |  | Port lightning protection：4KV 8/20us； Protection Level：IP30 |  |  |
| Safety Certification |  | CE,FCC,RoHS |  |  |
| Warranty Period |  | 3 years |  |  |

【Product Dimension】

# 【Packing List】

| Packing List | Item Name | Quantity | Unit |
| --- | --- | --- | --- |
|  | 16+2+1F Full Gigabit Smart POE Switch | 1 | set |
|  | AC Power Cord | 1 | piece |
|  | Quick start guide | 1 | piece |
|  | Warranty Card and Certificate of Conformity | 1 | piece |

# 【Ordering Information】

| Equipment Name | Equipment Information | Standard Power Supply |
| --- | --- | --- |
| MWS-SG1621Q | Unmanaged 18 10/100/1000M network ports + 1 gigabit SFP optical port slot PoE switches, ports 1-16 support IEEE 802.3af/at international standards, support PoE watchdog automatic restart, support port VLAN isolation, support 10m/ 250m long-distance transmission, built-in power supply for desktop and 1U rack mounting | Input: AC100-240V Output: DC52V |

# 【Option Information】

| Name | Model number | Descriptions | Unit |
| --- | --- | --- | --- |
| 1.25 G optical module | SFP-1.25G-SM1310/1550-SC | SFP optical module, 1.25G, single-mode single-fibre SC interface, transmission distance 3/20/40/60/80/100/120km optional, support for DDM function, support for hot-swap. | pair |
|  | SFP-1.25G-SM1310/1550-LC | SFP optical module, 1.25G, single-mode single-fibre LC interface, transmission distance 3/20/40/60/80/100/120km optional, support DDM function, support hot-swap. | pair |
|  | SFP-1.25G-LX-SM1310 | SFP optical module, 1.25G, single-mode dual-fibre LC interface, transmission distance 3/20/40/60/80/100/120km optional, support for DDM function, support for hot-swap. | piece |
|  | SFP-1.25G-SX-MM850 | SFP optical module, 1.25G, multi-mode dual-fibre LC interface, transmission distance 550M, support DDM function, support hot-swap. | piece |
| Electrical Port Module | SFP-1000Base-T-100M | 1.25G SFP optical module to 10/100/1000M RJ45 network port, (optical to electrical) | piece |

# 【Contact Us】

Shenzhen Meiweisi Technology Co.,Let

Business Hotline：

Technical Support：

E -Mail：

Website：

Adress：

About the document The trademarks, pictures and logos in this document are the property of Shenzhen Meiweisi Technology Co. Without the express written permission of Shenzhen Meivise Technology Co., Ltd, this document may not be copied, reproduced, transcribed or translated in any form or by any means (electronic, mechanical, photocopying, recording or any other possible means) for the purpose of dissemination of merchandise or for any commercial or profit-making purpose. This document may contain predictive information, so the information in this document is for reference only, does not constitute any offer or promise, the company to make corrections or modifications without notice. Copyright © Shenzhen Meiweisi Technology Co., Ltd